Job # 1

To prepare layout of strength of materials lab

- 10 ton Buckton UTM

- 50 ton Denison UTM

- Shimadzu 500kN tension testing machine

- Avery torsion testing machine

- Charpy impact testing machine

- 100 ton Buckton UTM

- Models of beams

- Avery and Shimadzu Rockwell hardness testing machine

- Brinell hardness testing machine

10. Shimadzu fatigue testing machine

11. Samples of bricks and tiles

Some important terms:

Layout:

It is a drawing or plan which shows the arrangement of various objects.

Universal testing machine:

Any machine which can perform tension, compression and bend test.

Hardness:

Resistance to plastic deformation or penetration is known as hardness.

Fatigue:

Repeated loadings which causes reversal of stresses in material is known as fatigue.

Machines in laboratory:

- 10 ton Buckton UTM:

Max. capacity: 10 tons

Manufacturer: Joshua Buckton and CO. LTD.

Operation: Electric/ manual lever arm system

Parts:  Upper part fixed and Lower part moveable

Tests:

- Tension

- Compression

- Bending

- Shear

- Punching

- Torsion

- 50 ton Denison UTM:

Max. capacity: 50 tons

Manufacturer: Smal. Denison & son ltd.

Operation: Electric hydraulic system

Parts: Upper part is fixed, lower part is moveable

Tests:

- Tension

- Compression

- Bending

Scales:

0-5 tons                                       0- 25 tons

0-10 tons                                     0-50 tons

- 50 tons Shimadzu UTM:

Max. capacity: 50 tons

Manufacturer: Shimadzu

Operation: electric

Parts:

Tests:

- Compression

- Bending

- tension

- Avery torsion testing machine:

Max. capacity: 15000 lb in

Manufacturer: W. & T. AVERY LTD. BRIMINGHAM, ENGLAND

Operation: Electric

Parts: Fixed head and twisting head

Tests: torsion test

Scales:

0-1500 lb in                                  0-7500 lb in

0-3000 ln in                                  0-15000 lb in

- Charpy impact testing machine:

Max. capacity: 0-170 degree

Manufacturer: CHARPY LTD. ENGLAND

Operation: Manual

Tests:

- Impact test (tension)

- Impact test (bending)

- 100 ton Buckton UTM:

Max. capacity: 200,000 lb

Manufacturer: Joshua Buckton & CO. LTD.

Operation: Electric

Parts: Upper part is fixed & lower part is moveable

Tests:

- Torsion

- Compression

- Bending

Scales:

20,000 lb                                      200,000 lb

I ton = 20 lb

L.C = 2000 lb

- Models of different structural elements:

- Avery Rockwell hardness testing machine:

Max. capacity:

Manufacturer: W. & T. AVERY LTD. BRIMINGHAM, ENGLAND

Operation: Manual

Tests: Hardness test

Scales:

Scale for low carbon steel

Scale for high carbon steel

Penetrators:

Steel ball 1/16 in. dia for low carbon steel

Diamond cone for high carbon steel

- Brinell hardness testing machine:

Manufacturer: Brinella Poland

Operation: Manual

Tests: Hardness test

- Shimadzu fatigue testing machine:

Manufacturer: Shimadzu

Operation: Electric

Test: Fatigue test